If the Vela firmware is siup-velars-260415a.dat or an earlier version, you must first upgrade the recovery card before updating to the latest version.

The recovery card upgrade procedure is as follows:

- 1.Download the SD card upgrade tool from the official website. Download link:

- https://www.science-image.com/download/sd-card-flashing-tool

2.Download the image archive package from the official website. Download link:

https://drive.google.com/file/d/1T0ujz3fpnjJrgNHiiyyxj5-it-ehK7cF/view?usp=drive_link

3.Connect the computer and C1 hard drive via a Type-C to USB adapter, then place the image installation package in the root directory of the C1 hard drive. Do not rename the image file.

- 4.Reinsert the C1 hard drive into the Vela device and power it on.

- 5.Launch the Recover.exe file (SD card upgrade tool) on your computer.

- 6.Enter the IP address of the Vela device and click "Restore from C1 SSD".Ensure that the computer and the Vela device are on the same network segment.

- 7.When the prompt "Completed" appears, the recovery card upgrade is finished.It will take approximately 10 minutes.

- 8.Perform restoration on the Vela device.

- 9.With the power off, press and hold the “Restore” hole  and Do not release on the Vela device using a pin tool.

- 10.Supply power to the Vela device.

- 11.Release the pin tool after approximately 3 seconds, The TFT screen of the Vela device displays restoration prompts.

- 12.After approximately 10 minutes, the screen will show that restoration succeeded; you may then power off and restart the device.

- Then download the new firmware of Vela and DynaTools,here is the link:

- https://www.science-image.com/support/download

- Then upgrade the firmware with Dynahub (an individual tool in DynaTools ). Here is the video link:

https://drive.google.com/file/d/1EIvmCehjvj4FliVlWNMiRnxfQwswgrub/view?usp=drive_link